GSC-Ottawa Polygon Model FileFormat: Jim Craven & Alan Jones

============================================================

Version 1.0: 17 Feb 1992

A Simple Polygon File Format for Model Interchange

--------------------------------------------------

A polygon file format is proposed for model interchange for the

following reasons:

1.  the polygon format is easily plotted and programs exist to

accomplish this using X Windows and Uniras.

2.  any number of parameters such as conductivity or density

may be associated with each polygon.

3.  provides a simple basis for conversion to model formats for

use in forward modeling programs.

4.  programs exist to convert Madden's EMCDC model files, Wannamaker's

PW2D model files, and OCCAM2 output model files to the polygon

format (model->ply).

5.  program are under development to convert the polygon files to

input model files to the above, and other, 2D modelling codes

(ply->model).

At present, the polygon format proposed satisfies the requirements of

only 1D or 2D modeling. A keyword in the title line could specify that

the polygons are fully 3D and three coordinates would be read in for

every vertex.

The Ottawa polygon file is composed of two parts: the header and polygon

information, both of which are formed with 7-bit ASCII bytes, i.e.,

readable by any standard editor. The header is the first three lines

(or records) in the file.  These lines may contain keywords and values

associated with those keywords that may be used in plot scaling or

annotating etc.

The polygon information starts on the 4th line with parameters

associated with a half-space.  The following records refer to polygons

that either tile or overlap upon the half-space (note overlap rule

specified below).

--------------------------------------------------------------------------------

Header Information (3 lines)

----------------------------

The first line contains a string either for use as a plot

title or to refer to the original model file that was

converted to a polygon file.  The key character ``>'' appearing in

this first line indicates the next 4 characters are the azimuth of the

survey line.  Another key, ``PRF='' indicates the next string is a

filename containing site easting and northing information.  The keys

are separated from the title string with a ``!'' character.

For example, the first header line may appear as:

Model No 1.5 !>-020 PRF=line8.prf

which specifies model number 1.5 of the station listed

in line8.prf rotated to an azimuth of -20 degrees.

The second and third line are x- and y-axis label information

respectively and generally appear as

Distance (km)

Log (Depth [km])

The distances in the polygon file should all be in metres but the

keywords ``km'' and ``Log'' indicate that the distances should

be scaled appropriately for plotting purposes.

--------------------------------------------------------------------------------

The Half-Space

--------------

The fourth line in the file specifies the location and parameters

of a half-space on which the subsequent polygons are to be tiled

or overlapped.

An example half-space record is:

000  100000  5550000  1   1000.000   'RES'

Note that the fields in this record are all being read using

FORTRAN free-format.

The first field is a polygon id which is normally 0.

The second and third fields are the model bind point expressed as

an easting and northing.  When coupled with the azimuth, this bind point

fixes the model in space.

The next field specifies the number of parameters associated with the

half-space; in this case there is only one parameter.

The remaining fields are pairs of parameter values and free format

character parameter labels.  There is only one pair in this case, the actual

half-space  parameter value (100 ohm.m) and the 'RES' label indicating the

parameter is in fact a resistivity.

If the model file specifies both resistivities and densities for the half

space and for the polygons, then an example half-space record would be:

0  100000  5550000  2   1000.0   'RES'     2890.0  'DENS'

(NOTE: all units in the file shoudl be in S.I.)

--------------------------------------------------------------------------------

The Polygons

------------

Subsequent pairs of records in the polygon file describe individual

polygons.  The fields in both records are purely numerical and are read in

using free format integer and real reads.

An example is:

001 4 1    10358.6

0.      0.      44000.      0.      44000.   1000.          0.   1000.

The first record for a particular polygon specifies the id (001), number of

vertices (4), number of parameters (1) and the parameter values (10358.6). The

parameter labels are not given as they are assumed to be the same as for the

half-space and in the same order.

The second record is a clockwise list of y-z pairs of the vertex locations for

that polygon. The y-z vertex values are determined using the following rules:

y: the y coordinate is specified as an offset along the

profile azimuth from the bind point specified in the half-space record

z: the z value specifies depth and increases downward

The units, as has already been mentioned, are metres. The special y-coordinates

-1.e32 and +1.e32 denote minus (most westerly) and plus (most easterly or deepest)

infinity respectively. Thus a layer across the model space can be specified, e.g.,

for a 10 ohm.m layer from 10-15 km.

022 4 1    10.

-1.e32 10000.      1.e32 10000.      1.e32 15000.      -1.e32 10000.

The Enf-of-file mark ends input - there is nowhere specified how many

polygons need to be read in. Also, the polygon id numbers need not be sequential

nor need they be complete. So you can easily delete a polygon or add another

out-of-sequence using a text editor (the overlap rule applies).

Overlap Rule:

-------------

In the case of overlapping polygons, the last specified takes precidence.

--------------------------------------------------------------------------------

Sample polygon file

-------------------

The following is a complete polygon file specifying a 1000 ohm.m

half-space and 10 polygons that do not overlap.

OCCAM2 files: l18.grid3;inmodel3;model16; !>0090 PRF=l18.prf

Distance (km)

Depth (km)

0  100000  5550000  1   1000.000  'RES'

001 4 1    10358.6

0.      0.      44000.      0.      44000.   1000.          0.   1000.

002 4 1     6395.9

44000.      0.      46000.      0.      46000.   1000.      44000.   1000.

003 4 1     4213.1

46000.      0.      48000.      0.      48000.   1000.      46000.   1000.

004 4 1     3384.5

48000.      0.      50000.      0.      50000.   1000.      48000.   1000.

005 4 1     3476.2

50000.      0.      52250.      0.      52250.   1000.      50000.   1000.

006 4 1      840.6

52250.      0.      54500.      0.      54500.   1000.      52250.   1000.

007 4 1     4020.7

54500.      0.      56000.      0.      56000.   1000.      54500.   1000.

008 4 1     7906.8

56000.      0.      58000.      0.      58000.   1000.      56000.   1000.

009 4 1    10651.2

58000.      0.      60000.      0.      60000.   1000.      58000.   1000.

010 4 1    10034.6

60000.      0.      62000.      0.      62000.   1000.      60000.   1000.

--------------------------------------------------------------------------------

If you have any questions/problems/suggestions, please contact either of us.

All the translation/plotting codes are available on request.

Jim Craven & Alan Jones

Geological Survey of Canada

1 Observatory Crescent

Ottawa, Ontario,

CANADA  K1A 0Y3

tel: (613) 992-8242 (Jim)   or    (613) 992-4968 (Alan)

FAX: (613) 992-8836

email: craven@cg.nrcan.gc.ca     or    ajones@nrcan.gc.ca