.nr PS 12

.nr VS 14

.LP

.ds CH Rhoplus

.ds RH %

.DA

.EQ

gsize 12

delim $$

.EN

.ce 3

.B

RHOPLUS: Optimum Fits and Bounds for MT data, $rho sub a$ and $phi$

.R

John. R. Booker and Robert L. Parker

.sp 0.5

.PP

This program solves exactly the one-dimensional (1D) inverse

problem of magnetotellurics given a set of apparent resistivity

and phase data.  The theory is the same as $D sup +$

except that calculations are done in the complex $omega$

plane on the log of $c$, not $c$ itself (Parker and Booker, 1996).

Input data can be mixtures

of apparent resistivity ($rho sub a$) and

impedance phase ($phi$) at the same or different

frequencies; $rho sub a$ alone, or $phi$ plus

just one $rho sub a$.  The following

calculations can be done: (1) compute the best fit to the data

in the weighted 2-norm sense; (2) calculate the model in

the space of positive delta functions (D+) that achieves

this minimum misfit; (3) find the upper and lower

bound on either $rho sub a$ or $phi$ holding $chi sup 2$

fixed at some tolerance corresponding to a misfit above the

minimum possible; (4) Optimally repair tears in apparent

resistivity resulting from different instrumental or site gains

in non-overlapping frequency bands.

.PP

The program operates under a simple command line-interpreter

in which you can enter commands interactively.

If you prefer

to run the program from a file, you can re-direct a file

containing the command script to standard input or you can

you can use the interactive command that will switch from

keyboard input to reading commands from a named file.

The following is a typical command script.

A percent sign tells the

program to ignore any text following so that you

can disable any command by putting a % in the first position

or you can document input scripts like this one:

.TS

tab(	);

l l s s s s

l l s s s s

l l s s s s

l l s s s s

l l s s s s

l r r r r r

l r r r r r

l r r r r r

l r r r r r

l r r r r r

l r r r r r

l r r r r r

l r r r r r

l r r r r r

l l s s s s

l l l s s s

l l l s s s

l l l s s s

l l l s s s

l l l s s s

l l l s s s

l l l s s s

l l l s s s

l l s s s s.

data *	% Read data table after execute command

task *	% Read table of bounding tasks after data table

period	% Data and task table use period rather than frequency

model	% Output best-fitting delta function model

execute	% Begin calculations

.sp 0.5

463.699	21.64	-12.38	55.58	16.38

112.601	25.43	4.62	52.02	5.22

29.7637	36.55	3.22	41.26	-2.52

7.45151	21.74	2.76	25.80	3.64

1.79057	8.15	0.86	33.77	3.04	2  0

0.460978	9.14	1.32	57.78	4.16

0.106602	23.08	2.28	61.78	2.82

0.0266121	-34.26	2.68	55.58	2.24

0.00605895	67.33	9.52	49.64	4.04

0	% End of data table

.sp 0.5

400	rho	phi

100	rho	phi

30	rho	phi

5	phi

1	rho	phi

0.5	rho

0.1	rho	phi

0.03	rho	phi

0	% End of task table

.TE

.LP

\fBCommand Catalog\fR

.PP

Every command takes the following form: a command word,

which may always be abbreviated to its first four letters

and which may appear alone or be followed by one or more

spaces, then numbers or text.  The command word must start at

the beginning of the line; lines starting with a blank are

treated as comments and are ignored. Unrecognized commands

are also ignored. This may lead to unexpected

results if commands are read from a file and are skipped

because of typos. It has been

done this way to give an interactive user the opportunity to

re-enter a miss-typed command. If commands are repeated with

different options, the last

entry is the one that is obeyed. In the following, default values are

those the program assigns if you do not explicitly set them;

you may rely on these defaults as sensible choices in most

cases. All information enclosed in [ ] is user supplied;

information enclosed in { } is optional.

.sp 0.5

.LP

\fIGeneral\fR:

.XP

\fB?\fR: Print a glossary of commands to standard error

(usually the screen, unit 0, but can be made any unit

by changing the the constant \fIierr\fR in \fIblockdata iounit\fR).

There are several help screens. After each topic is printed you

are asked for input: a carriage return skips to the

next topic; a y prints the next topic;

r{esume} returns control to the command interpreter. The

command \fIhelp\fR is a synonym for \fI?\fR.

.XP

\fBexecute\fR: With the required commands and

options saved in memory, begin

the calculations for this run.  The program always finds the

best-fitting solution (the smallest $chi sup 2$) compatible

with the input data and reports the minimum misfit. If you

use the \fIbands\fR command, you can calculate optimum

gains in different data bands relative to the first band defined.

The \fImodel\fR command will additionally

calculate the corresponding best-fitting model in D+.

Finally, if requested in the \fItask\fR file, the program will also

find limits on acceptable data values at arbitrary frequencies.

An end-of-file is interpreted as an \fIexecute\fR command in

the case that the data and task are read from named files rather

than following the command script as in the the example above.

The command \fIcontinue\fR is a synonym for \fIexecute\fR.

.XP

\fBq{uit}\fR: Exit program. Can only

be invoked prior to the \fIexecute\fR command. This

command can be abbreviated to less than four letters.

.XP

\fBecho {choice}\fR: Turns echoing of commands

as they are executed {on} or {off}. However, you can enter

commands in any order, with the exceptions

that \fIexecute\fR must be last and \fIread\fR permanently transfers

reading of commands to a named file. The default

is \fIecho off\fR. The default if {choice} is blank

is \fIecho on\fR. If the general output goes to your screen,

there will sometimes be two echos.

.sp 0.5

.LP

\fIInput\fR:

.XP

\fBread [filename]\fR: Immediately transfers reading of commands

and options to the named file. The program terminates if it cannot

open the named file for reading. You can chain \fIread\fR commands,

but once invoked, you cannot return to the point you were at

when a read command was issued. This means that a read command that

points back to a file already read will generate an infinite loop.

.XP

\fBdatafile\fR [filename]: It is \fImandatory\fR to give the

program data and their errors at frequencies on which

the computations are to be performed. No $phi$ data are required,

but $rho sub a$ at at least one frequency is required.

The data can be provided in a text file called [filename].

Alternatively, you may enter an asterisk as in the example;  then

the data are read from the standard input, immediately after

the \fIexecute\fR command has been given. Blank lines before, after

or within the data are ignored. The format of each line of the file

is as follows:

.RS

.TS

tab(	);

c c c c c c c.

f	$rho sub a$	$delta rho sub a$	$phi$	$delta phi$	{ $dnwt ( rho sub a )$	$dnwt ( phi )$ }

.TE

.RE

where: f is the frequency in Hz of the data on the line (or the

period in seconds if the \fIperiod\fR command has been invoked);

$ rho sub a $ is the apparent resistivity in Ohm-m and $delta

rho sub a$ is its estimated uncertainty; $phi$ is the phase

in degrees and $delta phi$ is its estimated uncertainty.

Frequencies (or periods) must monotonically increase or decrease.

Uncertainties are assumed to be one standard deviation.

$phi$ is assumed to be in the first quadrant

(i.e. between 0 and 90 degrees).

.sp 0.5

The two down-weights at the end of the line are optional, but if one

is there, both must be there. The estimated uncertainties of each

datum on the line are \fImultiplied\fR by the appropriate down-weight

prior to further calculations.

The re-weighted errors are shown on any plot

generated. A down-weight of 0.0 is a special case that

means ignore this datum (i.e. is equivalent to an infinite

uncertainty). Negative errors are set to zero by the program and the

corresponding data are ignored in the computations.

Zero errors also cause the program to ignore the corresponding datum.

Finally, the program ignores any $rho sub a~

<=~0$ in all calculations.

.sp 0.5

Reading of data terminates when the program finds a zero frequency

or an end-of-file. Numbers following the initial 0 on a data line

are ignored as long as the 0 is followed by at least one blank.

.sp 0.5

It is worth noting that the algorithm actually fits

$log sub e ( rho sub a )$ and radian $phi$. Thus data are

internally converted to their logarithmic form

$delta log sub e ( rho sub a )~=~ delta rho sub a / rho sub a$ and

errors in $rho sub a$ are thus assumed to be log normally distributed.

The practical implications of the differences between normally

and log normally distributed errors are small unless errors are large.

.XP

\fBperiod\fR: Period in seconds is provided in the first position

of the data and task files instead of frequency.

.XP

\fBlogdata\fR: Invoking this option implies that the resistivity

data are $log sub 10 ( rho sub a )$ and their errors are

$delta log sub 10 ( rho sub a )~=~0.4343~delta rho sub a / rho sub a$.

As already noted, log data and errors are more fundamental to the algorithm.

Negative values of logarithmic data are permitted, but negative

errors still cause the program to skip the corresponding datum.

Using this option also causes parallel changes in some of the output.

.sp 0.5

.LP

\fIComputation\fR:

.XP

\fBsurface\fR [choice]: Makes the upper surface boundary condition

\fIcon{ducting}\fR

or \fIins{ulating}\fR. The default is \fIcon{ducting}\fR.

If you are warned that the program is trying

to reverse the surface condition, you should change it

and re-run the program. If neither or both choices result

in such a warning, you should use the choice that results in

the smaller $chi sup 2$.

.XP

\fBmodel\fR: Compute the best-fitting model in D+.

.XP

\fBtask\fR [filename]: Optionally, you may provide a file of

frequencies in any order at which the $phi$ or $rho sub a$

values consistent with the data are bounded above and

below. The level of misfit deemed acceptable is provided

through the \fIcriterion\fR command and must exceed the

minimum possible. The format of each line

of the task file is as follows:

.RS

.TS

tab(	);

c c c.

f	[{rho}	{phi}]

.TE

.RE

Again, f is the desired frequency at which the bound is

to be calculated (or the period, if the \fIperiod\fR command was invoked

to read in the original data).

One or both of the words \fIrho\fR or \fIphi\fR tell

the program the kind of bounds desired. The reading of the task

file is again terminated by zero frequency or by an end-of file.

.sp 0.5

Instead of a separate file, the tasks can be tabulated on standard input

at the end of the data, as in

the  example above; the [filename] is then an asterisk.

There can be blank lines between the observation and task tables,

but no other characters. There can also be blank lines within the

task input.

.sp 0.5

Alternatively, the word \fIimplicit\fR can be entered for the

[filename]; then the program looks for frequencies in the input

data at which any of the measurements are missing (have uncertainties

$<=~0.0$ or $rho sub a~<=~0$ or have been

excluded by setting a down-weight to zero)

and finds the bounds on those variables.

.XP

\fBcriterion\fR [#]: In the bounding exercise, a level of

tolerable misfit to the included input data is needed.

[#] is the target value of either $chi sup 2$ or $1-q$, where

$q$ is the probability

that the target misfit will be exceeded by chance.

The second option is understood if $0~<~[#]~<1$.

By default, the program assumes a target value of $chi sup 2$

corresponding to $1-q$ = [#] = 0.95, the 95% confidence level.

.XP

\fBbands\fR [2N numbers] {-#} {p{runephase}}: This command

allows you to specify the

frequency indices of the bottom and top of up to four

bands. The bands need not be in ascending order or

adjacent, but they must not overlap.

If there are N bands, there will be 2N indices.

Only $rho sub a $ data inside these bands will be used

for computations. Whether $phi$ data are also eliminated outside

the bands depends on the option {-#} and whether the argument

{prunephase} (which can be abbreviated to as short as

its leading letter) is present. The

indices correspond to the data frequencies (periods) in the

order that they are given in the data file. Thus 1 is the first

frequency, etc. The first band you define need not start with

1. For example \fIbands 10 19 2 8 23 29\fR defines three

bands within at least 29 data frequencies.

.sp 0.5

The option number {-#}

(which has a default value of 0) enables

estimation of the average gain (the number you multiply

times the $rho sub a$) in each band

that minimizes the misfit of the best-fitting model.

Using this feature one can, for instance,

determine the optimum merge of data taken at the same site by

a long period and a wideband system. One can also test for

incorrect gain calibrations in different bands of a wideband system.

.sp 0.5

The \fIbands\fR command assumes that the gain in

the first band defined will be held fixed (at 1 unless set otherwise

by the \fIgains\fR command). The program computes the optimum

gain of the next band relative to the first and then

the next with respect to the first two after band two is corrected

and so forth. Thus the first band defined

is the one that is assumed to have the correct gain.

The order in which the bands are incorporated is completely

flexible aside from the requirement that the bands not overlap.

(Note: there is a flag that you can set in \fIsubroutine gtband\fR

that tells the program to use only the $rho sub a$ in the first

band and the band whose gain is currently being estimated rather than

all the previously corrected $rho sub a$. See comments in that

subroutine.)

.sp 0.5

The option {-#} controls the specifics of how the gains are computed:

.RS

.IP $>=0$

All gains are held fixed. This is simply an alternate way to limit

which data are included in any computations and is the default if

the final option number is missing or has an illegal value.

This option automatically excludes all phase data outside of the

defined bands.

.IP -1

The optimum gain is found by shifting all the $rho sub a$ in the

band until the misfit to all the data currently included is

minimized. For the gain in the second band, this means all

the $rho sub a$ data in the first two bands that have not been

otherwise excluded, plus all the included $phi$ data.

.IP -2

The optimum gain is found by shifting all the $rho sub a$ in the

band until the misfit to the $rho sub a$ data currently

included is minimized. Subsequent tasks are done with all phase

excluded. This is equivalent to option -1 with all phase down-weights

set to 0.

.IP -3

Optimum gains are computed as in option -2, but

in subsequent bounding tasks, phases not otherwise excluded

are included again.

.LP

Additional options -4 and -5 exist, but will probably be of little

interest. Their effect is described

in comments at the beginning of \fIsubroutine gtband\fR.

All options that enable gain estimation require the argument

\fIprunephase\fR if you want to exclude $phi$ outside the

defined bands. If this argument is not present, all phase data

are used unless excluded by 0.0 down-weights or non-positive errors.

.RE

.XP

\fBgains\fR [N numbers]: This command modifies the effect of the

\fIbands\fR command. It requires N numerical arguments, where

N is the number of bands defined by \fIbands\fR. Each number

multiplies $rho sub a$ (or is \fIadded\fR to $log sub 10 ( rho sub a )$

in the case that \fIlogdata\fR is invoked). The order of the gains

is the same as defined by \fIbands\fR. The gains are

applied prior to any further calculations. The gain applied to the

first band is subsequently held fixed. If the gain is \fInot\fR 1.0

(0.0 for \fIlogdata\fR) in any subsequent band, the gain in

that band immediately becomes frozen and will not be estimated.

For example, if $logdata$ is in force and the bands are

defined by \fIbands 10 19 2 8 23 29 -1 prune\fR, the command

\fIgains\fR .301 0. 1. implies that the log resistivities in

band 10-19 will be increased by 0.301 (equivalent to multiplying the

$rho sub a$ by 2.); then the gain in band 2-8 will be estimated

using the modified resistivities in band 10-19 and the phases within

all the bands; band 23-29 will have its

log 10 resistivity increased by 1. (equivalent to multiplying

$rho sub a $ by 10.), but its gain will \fInot\fR be estimated. Because

it was the final band defined, the modified resistivities

in band 23-29 are not used in estimating the gain of

band 2-8, but will be used in any subsequent bounding tasks.

Because \fIprune\fR is present, all calculations

ignore phase data between the bands (i.e. 1, 9 and 20-22).

.XP

\fBnlambda\fR [#]:

The program reports the number of samples on the

imaginary axis of the $omega$ plane following

the data read from the input file.

You can force the program to sample the imaginary

axis at [#] points instead of the number

it chooses with the aid of an internal formula.

You may want more accuracy (larger [#]) or more speed (smaller [#]).

In general, you should use the smallest value of [#] that you can

get away with, because execution time rises rapidly with the number

of samples. The default is a compromise that should result in 2.5

significant figures of accuracy in the responses,

which is comparable to the very

best data.

.LP

\fIOutput\fR:

.XP

\fBroot\fR [name]: This command specifies a root name to be used in

constructing output file names for the general output file

and the optional output files associated with the commands

\fItask\fR, \fIplot\fR, \fImatrix\fR

and \fIdebug\fR according to the following table:

.RS

.TS

tab (	);

l l l l.

Command	default file	root [name]

.sp 0.5

general	standard output	name.out

task	rplus.bnd	name.bnd

plot	rplus.plt	name.plt

matrix	rplus.rsp	name.rsp

debug	rplus.dbg	name.dbg

.TE

.RE

The default is used when [name] is not given or

\fIroot\fR is not invoked.

.XP

\fBprintlevel\fR [#]: Various amounts of information can be is written

to standard output

by setting [#] to integers from 0 to 5, more output as the

integer rises. Even with the default of zero, you will have

everything important. When [#] $>=$ 4 additional output is written to

the debugging file defined above.

The command \fIdebug\fR is completely equivalent to [#] = 5.

.XP

\fBplot {option}\fR: Calls a user-defined subroutine

to write a plotting program script to summarize the results.

Two options are implemented:

.RS

.XP

\fIplotxy\fR - Bob Parker's plotxy program as implemented at U.C.S.D.

.XP

\fIxyplot {fancy}\fR - The Chave/Schultz version of plotxy implemented

at the U.W. If the argument "fancy" follows the option, the bounds (if

computed) are shown as upward and downward triangles. If this text

is missing, the bounds are simply connected by dotted lines. It

is important to understand that the line connecting the bounds implies

nothing about the actual bound between points where bounds are calculated.

Whether or not the plot is fancy, original data are shown as filled

circles if they are included in the computation and open circles

if they are not. Error bars on included data are modified by the

down-weight values, while excluded data have unweighted error bars.

The responses of the best-fitting model at the frequencies of the

data are connected by a solid line.

.RE

A dummy subroutine, which should help you implement your own plot

option, is provided. Read the comments at the start of the main

program.

.XP

\fBmatrix\fR: Write out two files in generic matrix format suitable

for direct importation to programs such as Matlab.

.sp 0.5

.bp

The first file with suffix .rsp has format:

.RS

.TS

tab(	);

c c c c c c c.

f	$rho sub a$	$delta rho sub a$	$rho sub a sup predicted$	$phi$	$delta phi$	$phi sup predicted$

$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$

- or -

.sp 0.5

f	$log sub 10 ( rho sub a )$	$delta log sub 10 ( rho sub a )$	$log sub 10 ( rho sub a ) sup predicted$	$phi$	$delta phi$	$phi sup predicted$

$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$	$cdot cdot$

.TE

.RE

Measured data are written as originally read (either $rho sub a$ or

$log sub 10 ( rho sub a )$; corresponding errors are

as modified by any down-weights; predicted responses

for the best-fitting model are given

at all frequencies (including those at which the errors are

zero and hence the data are excluded).

.sp 0.5

The second file with suffix .bnd has format:

.RS

.TS

tab(	);

c c c.

# of $phi$ bounds	1-q	${chi sup 2} sub min~~$

f	$phi sub LB$	$phi sub UB$

$cdot cdot$	$cdot cdot$	$cdot cdot$

# of $rho sub a$ bounds	1-q	${chi sup 2} sub target$

f	$rho sub LB$	$rho sub UB$

$cdot cdot$	$cdot cdot$	$cdot cdot$

.TE

.RE

where the subscripts refer to lower and upper bounds, $q$ is

defined in the \fIcriterion\fR command and the $chi sup 2$

are the minimum achieved by the best-fitting model

and the target value for the reported bounds. The resistivity

bounds will be $log sub 10$

if the \fIlogdata\fR option is in force.

.XP

\fBdebug\fR: This option is supplied in case you want

output with information useful for

debugging written to the debugging file defined in the

\fIroot\fR command. This command

automatically sets the \fIprintlevel\fR to 5.

.sp 0.5

.LP

\fBNotes\fR

.PP

Error messages will be generated if obvious things are

incorrect, such as the absence of a data file, or when the

target misfit in the bounding part of the program is less

than the minimum possible. Warnings are printed when things

may not be working as you may have intended. A warning is also

printed when the

iterative solution of the bounding problem fails. At the

end, the failed bounds are noted in the summary table.

.PP

The program runs in less than 1.5 MB as dimensioned.

Execution time is dominated by solution of the Bounded

Value Least Squares (BVLS) problem. Thus finding the

best-fitting model is very fast. Finding optimum gains

or bounding data are much slower because they are iterative

and a BVLS problem  must be solved at each iteration.

The BVLS code responds well to compilation optimization and

thus optimization can reduce execution time dramatically.

.sp 0.5

.LP

\fBCase Study\fR

.PP

Figures 1 to 3 illustrate application of many of the

features of this program to the problem of detecting and

correcting probable data inconsistencies in responses from a

single dipole of

an EMAP collected in south-central Oklahoma. Although the data

are not 1-D, the dipole in question is

assumed to be nearly perpendicular to strike and thus should

be B-Polarization (TM mode). Weidelt and Kaikkonen (1994)

show that 2D responses for this

polarization fall within the space spanned by all 1D responses.

More generally, Rhoplus provides a way of fitting a response

to noisy data that is guaranteed to have a physically acceptable

(causal) relationship between $rho sub a$ and $phi$. Although

E-polarization (TE) and 3D data cannot be guaranteed to have

a $rho sub a~-~phi$ relationship that falls within the set spanned

by 1D responses, the situations in which they do not are

rare enough that it is always worth trying the techniques

that we describe on non-1D data.

.PP

A table of the input used to generate

the results in Figure 3 is appended. The results

in Figures 1 and 2 can be generated by suitable editing.

These data were provided by Exxon courtesy of Leonard Srnka and

are one of the examples in Parker and Booker (1996).

.PP

The program is first run on all the data. The minimum

possible $chi sup 2$ of 91.46 exceeds the 95% confidence

target and so no bounds can be calculated. Note that

the data with periods greater than 0.3 s are well-fit. The

large $chi sup 2$ is clearly coming from the shorter period data

and involves a systematic bias of the fit to $rho sub a$ and

a systematic trend to the misfit to the predicted $phi$.

.PP

Figure 2 fits only the phase data and one of the well-determined

longer-period resistivities. The $chi sup 2$ is now small enough to

estimate bounds. However, the only bound requested was

for the phase at 4.5 Hz (0.22 s). Inspection of Figure 1 suggested

that this phase datum might be an outlier despite the fact

that the predicted phase response fortuitously runs though its

error bar. Figure 2, which was generated with \fIplot xyplot fancy\fR,

clearly allows one to conclude that this datum is an outlier

in the context of the complete phase data set. Not only does its

most probable value lie below the lower bound, but its

range within its error estimate also lies almost entirely below

the bound. The excellent

fit to the rest of the phase and most of the resistivities

plus the obvious systematic bias

of the predicted $rho sub a$ with respect to the eight

high frequency data,

clearly suggests a tear in the resistivity data between 3.0

and 4.5 Hz.

.PP

Both $rho sub a$ and $phi$ at 4.5 Hz were excluded from the

computation shown in Figure 3. Excluding a suspected

outlier leads to more generous bounds if its exclusion

decreases the $chi sup 2$ of the best-fitting model.

Thus, a datum that falls outside these more

generous bounds can be judged inconsistent with more confidence.

.PP

In Figure 3, the optimum gain for the seven highest frequency

$rho sub a$

was estimated using option -1. The result is a gain of

1.344 and a large

reduction in the $chi sup 2$ relative to Figure 1.

Bounds for resistivity and phase were

estimated at all data frequencies and plotted without the fancy

option. The single excluded phase remains an outlier, but there

is clearly no reason to exclude the resistivity at the same period.

.PP

You should try the following exercises: Bound all $rho sub a$

using only the $rho sub a$ data and then estimate the high frequency

gain and bounds to all $rho sub a$ and $phi$ using options -2.

The first exercise should lead you to conclude that the

resistivity at 4.5 Hz is an outlier with respect to the $rho sub a$

data prior to any gain correction and that the $rho sub a$ with

no gain modification are

compatible with B-polarization (TM). However, this

requires accepting that the phase data are either incompatible

with the resistivities or have very significantly underestimated

error bars at frequencies above 1.5 Hz.

The second exercise (option -2) results in almost exactly same gain

as before if the resistivity at 4.5 Hz is

included and 1.2185 if it is excluded. However, the bounds when

$rho sub a$ at 4.5 Hz is excluded do not support its exclusion.

Recomputing the bounds with the gain of 1.2185 and phase

included (option -3) leads to a credible scenario in which both

$rho sub a$ and $phi$ at 4.5 Hz can be excluded. $chi sup 2$ is somewhat

higher than for Figure 3 (17.26 versus 10.55) but still acceptable.

This illustrates the difficulty of estimating gain with resistivity

alone. Because phase is less prone to bias than is

resistivity, we favor using both resistivity and phase unless there

is a strong argument against it.

.sp 0.5

.LP

\fBReferences\fR

.XP

Parker, R.L. and J.R. Booker,

Optimal One-Dimensional Inversion and Bounding

of Magnetotelluric

Apparent Resistivity and Phase Measurements,

\fIPhys. Earth Planet. Int, 98\fR, 269-282, 1996.

.XP

Weidelt, P., and P. Kaikkonen,

Local 1-D interpretation of magnetotelluric B-polarization impedances,

\fIGeophys. J. Internat., 117\fR, 733-748, 1994.

.sp 0.5

.LP

\fBInput to generate results in Figure 3\fR

.TS

tab (	);

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s

l l s s s s s.

echo on 	% echo the commands

data * 	% Read data table at end of this file

task * 	% Read table of bounds from end of this file

bands 9 26 1 8 -1 	% find optimum offset of band 1-8 with 9-27 held fixed

root fig_3 	% Root for file names

surface insul 	% Use insulating BC at surface

criterion 0.95 	% Misfit goal for tasks corresponds to 95% confidence

plot xyplot 	% make a plot file of given name

model 	% Compute delta function model

matrix 	% output in matrix form

execute 	% Begin calculations

.TE

.TS

tab (	);

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l l l l l l

l l s s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l l s s s s

l l s s s s s.

48.000     	6.8780    	0.31550    	48.500    	 1.3000	1	1

36.000     	6.5140    	0.26050    	44.700    	 1.1500	1	1

24.000     	6.4490    	0.30100    	45.800    	 1.3000	1	1

18.000     	6.5240    	0.32900    	44.600    	 1.4500	1	1

12.000     	6.7140    	0.24600    	46.700    	 1.0500	1	1

9.0000     	6.7080    	0.30250    	44.900    	 1.3000	1	1

6.0000     	6.7860    	0.45550    	45.700    	 1.9000	1	1

4.5000     	5.9590    	0.38900    	39.000    	 1.9000	0	0

3.0000     	7.8370    	0.14050    	41.500    	0.50000	1	1

2.2500     	7.9180    	0.15200    	38.100    	0.55000	1	1

1.5000     	8.6770    	0.23050    	33.700    	0.75000	1	1

1.1250     	9.7540    	0.29100    	30.600    	0.85000	1	1

0.75019     	11.600    	0.69000    	27.800    	 1.7000	1	1

0.56243     	13.660    	0.57500    	26.600    	 1.2000	1	1

0.37495     	17.400    	 1.8450    	24.800    	 3.0500	1	1

0.28129     	21.360    	 1.9150    	25.700    	 2.5500	1	1

0.18751     	25.610    	 3.5250    	26.000    	 3.9500	1	1

0.14061     	30.650    	 2.3000    	30.500    	 2.1500	1	1

0.93721E-01 	33.150    	 3.3450    	34.600    	 2.9000	1	1

0.70323E-01 	33.940    	 2.5400    	43.000    	 2.1500	1	1

0.46882E-01 	36.650    	 4.8550    	45.300    	 3.7500	1	1

0.35162E-01 	35.040    	 4.2600    	48.700    	 3.5000	1	1

0.23441E-01 	32.040    	 4.3200    	54.000    	 3.8500	1	1

0.17581E-01 	30.020    	 4.1500    	56.600    	 3.9500	1	1

0.11721E-01 	26.970    	 3.8500    	61.300    	 4.1000	1	1

0.87873E-02 	24.920    	 3.6950    	63.900    	 4.2000	1	1

0 	% End of data table

.sp

48.000     	rho 	phi

36.000     	rho 	phi

24.000     	rho 	phi

18.000     	rho 	phi

12.000     	rho 	phi

9.0000     	rho 	phi

6.0000     	rho 	phi

4.5000     	rho 	phi

3.0000     	rho 	phi

2.2500     	rho 	phi

1.5000     	rho 	phi

1.1250     	rho 	phi

0.75019     	rho 	phi

0.56243     	rho 	phi

0.37495     	rho 	phi

0.28129     	rho 	phi

0.18751     	rho 	phi

0.14061     	rho 	phi

0.93721E-01 	rho 	phi

0.70323E-01 	rho 	phi

0.46882E-01 	rho 	phi

0.35162E-01 	rho 	phi

0.23441E-01 	rho 	phi

0.17581E-01 	rho 	phi

0.11721E-01 	rho 	phi

0.87873E-02 	rho 	phi

0     	% End of task table

.TE

.sp

.nf

Robert L. Parker

rlparker@ucsd.edu

.sp

John R. Booker

booker@geophys.washington.edu